MATERIAL SAFETY DATA SHEET

材料安全数据表

Section 1: Identification of the substance/mixture and of the company/undertaking
1：物质说明/混合物和公司/企业识别

1.1 Product identifier

1.1 产品标识
Product name:Chlorinated Polyethylene

产品名称：氯化聚乙烯
Trade name:CHLORINATED POLYETHYLENE CPE135A

商品名称：CPE135A
Chemical attribute:Long molecular chain belt organic chemicals.

化学属性：高分子有机化合物
REACH Reg. No.:No registration number is given yet, since this substance is a polymer and exempted from registration according to article 2.9 in REACH Regulation.

REACH 标准：还没有注册号，因为这种物质是聚合物，并根据第2.9条的规定，在达到规定的注册豁免

1.2 Relevant identified uses of the substance or mixture and uses advised against
Identified uses:Used in the plastics industry as an additive to modify a range of properties. Also used in rubber industry.
ses advised against:Not available.

1.2产品用途：塑料制品的改性剂，也可用于橡胶制品。

1.3 Details of the supplier of the SDS

1.3公司信息
Manufacturer:SHANDONG SANYI INDUSTRIAL CO.,LTD

公司名称：山东三义实业股份有限公司
Address:XIAXI ROAD, DONGCHENG INDUSTRIAL PARK, LINQU, WEIFANG, SHANDONG,CHINA

地址：中国山东省潍坊市临朐县东城经济开发区夏西路

E-mail:export@sanyichemical.com

邮箱：export@sanyichemical.com
Telephone:0086-536- 3353988

电话：0086-536- 3353988
Fax:0086-536-3353966

传真：0086-536-3353966

1.4 Emergency telephone number

1.4急救电话
in China: +86-536-3117168(only office hours available.)

+86-536-3117168（办公时间可用）

Section 2: Hazards identification

2：危害说明

2.1 Classification of the substance or mixture
Classification according to Regulation (EC) No 1272/2008[CLP]

2.1物质分类

依据(EC) No 1272/2008[CLP]分类
This product does not meet the criteria for classification in any hazard class according to Regulation (EC) No 1272/2008 on classification, labelling and packaging of substances and mixtures

本产品在(EC) No 1272/2008中没有任何危险识别的分类，包括标签和包装。
Classification according to Directive 1999/45/EC

按1999/45/EC分类
This product does not meet the criteria for classification in any hazard class according to Directive 1999/45/EC on classification, labelling and packaging of substances.

本产品在1999/45/EC中没有任何危险识别的分类，包括标签和包装。

2.2 Label elements

2.2标签元素
Labelling according to Regulation (EC) No 1272/2008 [CLP]

依据(EC) No 1272/2008 [CLP]
No labelling information about hazard pictogram, signal word, hazard statements and precautionary statements available. This product does not meet the criteria for classification in any hazard class according to Regulation (EC) No 1272/2008 on classification, labelling and packaging of substances and mixtures.

没有标签信息，标识词和灾害预防报表可用。此产品不符合（EC）1272 / 2008中规定的有关任何危险类别、标签和包装的分类。
Labelling according to Directive 1999/45/EC

依据1999/45/EC

No labelling information about symbol, risk phrases and safety phrases available. This product does not meet the criteria for classification in any hazard class according to Directive 1999/45/EC

没有标签信息，危险短语和安全短语可用。此产品不符合（EC）1272 / 2008中规定的有关任何危险类别的分类。

2.3 Other hazards

2.3其他危险
No information available.
无可用信息

Section 3: Composition/information on ingredients

- 成分/信息
3.1 Substance information

- 3.1物质信息

| Substance name 物质名称 | CAS No. CAS编号 | EC No. EC编号 | Classification according to 67/548/EEC 67/548/EEC分类 | Concentration 含量 |
| --- | --- | --- | --- | --- |
| Chlorinated polyethylene 氯化聚乙烯 | 64754-90-1 | Not available 不可用 | Not classified. 不可用 | ≥ 95.0 % |
| Calcium distearate 硬脂酸钙 | 1592-23-0 | 216-472-8 | Not classified. 不可用 | ≤ 2.5 % |
| Talc (Mg3H2(SiO3)4) 滑石粉 | 14807-96-6 | 238-877-9 | Not classified 不可用 | ≤ 2.5 % |

| Substance name 物质名称 | CAS No. CAS编号 | EC No. EC编号 | Classification according to Regulation (EC) No 1272/2008 No 1272/2008 分类 | Concentration 含量 |
| --- | --- | --- | --- | --- |
| Chlorinated polyethylene 氯化聚乙烯 | 64754-90-1 | Not available 不可用 | Not classified. 不可用 | ≥95.0% |
| Calcium distearate 硬脂酸钙 | 1592-23-0 | 216-472-8 | Not classified. 不可用 | ≤ 2.5 % |
| Talc (Mg3H2(SiO3)4) 滑石粉 | 14807-96-6 | 238-877-9 | Not classified 不可用 | ≤ 2.5 % |

Section 4: First aid measures

- 急救措施
4.1 Desc ription of first aid measures

- 4.1描述
General notes:In all cases of doubt, or when symptoms persist, seek medical attention.

- 一般事项：所有危险情况下，应寻求医疗救护
Following inhalation:

- 吸入：
Remove from exposure and move to fresh air immediately. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. If effects occur, consult a physician.

- 立即转移到安全地区，必要时进行人工呼吸。呼吸困难时输氧气，询医。

Following skin contact:

接触皮肤：

Wash skin with plenty of water. Seek first aid or medical attention as needed. If molten material comes in contact with the skin, do not apply ice but cool under ice water or running stream of water. DO NOT attempt to remove the material from skin. Removal could result in severe tissue damage. Seek medical attention immediately.

用大量的水冲洗皮肤。依据需要寻求急救或医疗救助。如果熔融物接触，用凉水或流动的水冲洗。禁止去掉皮肤上的熔融物。否则可能会导致严重的组织损伤。并立即医疗救助。
Following eye contact:

接触眼睛：
Flush eyes thoroughly with water for several minutes. Remove contact lenses after the initial 1-2 minutes and continue flushing for several additional minutes. If effects occur, consult a physician, preferably an ophthalmologist. May cause injury due to mechanical action.

用清水彻底冲洗。摘掉眼镜，继续冲洗，询医，否则可能会导致眼睛损伤。
Following ingestion:

遵循：
If swallowed, seek medical attention. May cause gastrointestinal blockage. Do not give laxatives. Do not induce vomiting unless directed to do so by medical personnel.

如果吞食，寻求医疗救助。这可能会导致胃肠道堵塞。不要吃泻药。不要催吐，除非是由医务人员这样做。
Notes for the doctor:

医辽提示：
If hydrogen chloride is liberated due to thermal degradation, treat as hydrogen chloride exposure. If burn is present, treat as any thermal burn, after decontamination. No specific antidote. Treatment of exposure should be directed at the control of symptoms and the clinical condition of the patient.

热分解会释放氯化氢气体，此时，像热烧伤一样的措施，没有特别的解毒药品。应根据患者的症状和临床状况询医。

4.2 Most important symptoms and effects, both acute and delayed

4.2最重要事项：
Potential Health Effects:

潜在的影响：
Eye Contact:

接触眼睛：
Solid or dust may cause irritation or corneal injury due to mechanical action. Thermal degradation of the resin may generate hydrogen chloride gas at concentrations which may cause eye irritation.

固体或灰尘和树脂的热分解产生的氯化氢会产生刺激造成角膜损伤
Skin Contact:

接触皮肤：
Prolonged contact is essentially nonirritating to skin. Mechanical injury only. Under normal processing conditions, material is heated to elevated temperatures; contact with the material may cause thermal burns. No adverse effects anticipated by skin absorption.

接触基本不刺激皮肤。在正常情况下，加热的材料与皮肤接触可能会导致烫伤。皮肤吸收没有可用的影响结果。
Inhalation:

吸入：
Dust may cause irritation to upper respiratory tract (nose and throat). Thermal degradation of the resin may generate hydrogen chloride gas at concentrations which may cause respiratory irritation.

灰尘可能会对上呼吸道（鼻和喉咙）造成刺激。树脂的热降解可能会产生氯化氢气体，这可能会导致呼吸道刺激。
Ingestion:

食入：
Very low toxicity if swallowed. Harmful effects not anticipated from swallowing small amounts. May cause choking if swallowed.

非常低的毒性。无可用的影响结果。吞食可能会导致窒息。

4.3 Indication of the immediate medical attention and special treatment needed

4.3特殊治疗提示
Persons with pre-existing skin, eye, or respiratory disease may be at increased risk from the irritant or allergic properties of this material. Attending physician should treat exposed patients symptomatically.
接触皮肤，眼睛，或吸入可能产生会有刺激或过敏。
Section 5: Fire-fighting measures

5：消防措施
5.1 Extinguishing media

5.1  灭火
Suitable extinguishing media:

灭火介质

ater fog or fine spray. Dry chemical fire extinguishers. Carbon dioxide fire extinguishers. Foam.

干粉，二氧化碳，泡沫
Unsuitable extinguishing media:

不适应的灭火介质
No information available.

不可用

5.2 Special hazards arising from the substance or mixture
Unusual Fire and Explosion Hazards:

5.2特殊火灾和爆炸危险：
Do not permit dust to accumulate. When suspended in air dust can pose an explosion hazard. Minimize ignition sources. If dust layers are exposed to elevated temperatures, spontaneous combustion may occur. Dense smoke is emitted when burned without sufficient oxygen.

禁止灰尘积聚。悬浮在空气中的灰尘会造成爆炸危险。如果粉尘层暴露在高温下可能会自燃。燃烧时没有足够的氧气时会产生浓烟。
Hazardous Combustion Products:

有害的燃烧产物：
During a fire, smoke may contain the original material in addition to combustion products of varying composition which may be toxic and/or irritating. Combustion products may include and are not limited to: Carbon monoxide. Carbon dioxide. Hydrogen chloride.

在火灾中，烟雾可能含有不同成分的燃烧产物，可能是有毒的或刺激性的。燃烧产物可能有一氧化碳，造成中毒，还有二氧化碳。氯化氢。

5.3 Advice for fire-fighters

5.3消防建议
Fire Fighting Procedures:

消防程序：
Keep people away. Isolate fire and deny unnecessary entry. Soak thoroughly with water to cool and prevent re-ignition. If material is molten, do not apply direct water stream. Use fine water spray or foam. Cool surroundings with water to localize fire zone. Hand held dry chemical or carbon dioxide extinguishers may be used for small fires. Dust explosion hazard may result from forceful application of fire extinguishing agents.

远离。用水浸泡，冷却，防止再生火。如果材料是熔融的，不适用于直接用水。使用细水雾或泡沫。手持干粉或二氧化碳灭火器可用于小火。
Special Protective Equipment for Firefighters:

消防专用的防护设备：
Wear positive-pressure self-contained breathing apparatus (SCBA) and protective fire fighting clothing (includes fire fighting helmet, coat, trousers, boots, and gloves). If protective equipment is not available or not used, fight fire from a protected location or safe distance.

佩戴正压自给式呼吸器（SCBA）和保护消防服装（包括消防头盔、上衣、裤子、靴子和手套）。如果没有保护设备，一定在安全距离扑灭火灾。
Section 6: Accidental release measures

6：泄露处理

6.1 Personal precautions, protective equipment and emergency procedures

6.1个人防护用品，防护设备和应急程序
Spilled material may cause a slipping hazard. Use appropriate safety equipment.For additional information, refer to Section 8, Exposure Controls and Personal Protection.

泄漏可能会导致打滑的危险。使用适当的安全设备。对于额外的信息，参见第8节，曝光控制和个人防护。

6.2 Environmental precautions

6.2环境注意事项
Prevent from entering into soil, ditches, sewers, waterways and/or groundwater. 
See Section 12, Ecological Information.

禁止进入土壤、沟渠、下水道、水道和下水道。见第12节，生态信息。

6.3 Methods and material for containment and cleaning up
6.3用于遏制和清理的方法和材料

Contain spilled material if possible. Sweep up.

打扫溢出的物质
Collect in suitable and properly labeled containers.

收集于适当的有正确标识的器具。
See Section 13, Disposal Considerations, for additional information.

参见第13部分，附加信息。

6.4 Reference to other sections

6.4其他参考部分
See Section 7 for information on safe handling.

参见第7部分有关安全的信息
See section 8 for information on personal protection equipment.

参见第8部分关于个人防护器材
See Section 13 for information on disposal.

参见第13部分关于处置的信息。

Section 7: Handling and storage

7：存储和处理

7.1 Precautions for safe handling

7.1安全处理注意事项

No smoking, open flames or sources of ignition in handling and storage area. Good housekeeping and controlling of dusts are necessary for safe handling of product. Avoid breathing process fumes. Use with adequate ventilation. When appropriate, unique handling information for containers can be found on the product label. Workers should be protected from the possibility of contact with molten resin. Do not get molten material in eyes, on skin or clothing. Pneumatic conveying and other mechanical handling operations can generate combustible dust. To reduce the potential for dust explosions, electrically bond and ground equipment and do not permit dust to accumulate. Dust can be ignited by static discharge.

禁止吸烟。良好的清洁和控制粉尘对产品的安全是必要的。避免呼吸过烟雾。使用有足够的通风的区域。在产品标签上找到唯一处理信息。人工保护防止熔融树脂接触的可能性。注意防护眼睛，禁止在皮肤或衣服上接触熔化的物质。气力输送和其他机械处理操作可产生可燃性粉尘。为了减少潜在的粉尘爆炸，地面设备不允许灰尘积累。静电放电可能点燃灰尘造成火灾。

7.2 Conditions for safe storage, including any incompatibilities

7.2安全存储的条件

Store in cool place.

存放于阴凉的地方。

Keep container tightly closed in a dry and well-ventilated place.

保持通风干燥 。

Store in accordance with good manufacturing practices.

养成良好的生产习惯。

Store away from incompatible materials (see Section 10).

禁止存储于不兼容的材料（参见第10部分）

7.3 Specific end use(s)

7.3特殊用途

Apart from the uses mentioned in section 1.2 no other specific uses are stipulated.

除了第1.2节中提到的用途外，没有规定其他具体用途

Section 8 : Exposure controls/personal protection

8：暴露控制/个人防护

| 8.1 Control parameters Chlorinated polyethylene CAS# 64754-90-1 |  | There are no currently occupational exposure limit values established. |  |  |
| --- | --- | --- | --- | --- |
|  | Talc CAS# 14807-96-6 |  | Australia: 2 mg/m3 respirable aerosol; Belgium: 2 mg/m3; Canada: 3 mg/m3; Denmark: 0.2 mg/m3 respirable aerosol; Hungary: 2 mg/m3 respirable aerosol; Spain: 2 mg/m3 respirable aerosol; Sweden: 2 mg/m3 inhalable aerosol; 1 mg/m3respirable aerosol; Switzerland: 2 mg/m3 respirable aerosol; The Netherlands: 0.25 mg/m3 respirable aerosol; USA - NIOSH: 2 mg/m3; United Kingdom: 1 mg/m3 respirable aerosol; |  |
|  | Calcium distearate CAS# 1592-23-0 |  | ACGIH - TWA: 10 mg/m3; USA - TLV: 10 mg/m3; Germany - MAK: 6 mg/m3; |  |
|  | Hydrogen chloride CAS# 7647-01-0 |  | MAC: 1 mg/m3; ACGIH: 2 ppm (Ceiling); OSHA Table Z-1: 7 mg/m3, 5 ppm (Ceiling); Hydrogen chloride may be generation under thermal degration conditions. |  |

8.1控制参数

Occupational exposure limit values:

暴露极限：

8.2 Exposure controls

8.2暴露控制

Appropriate engineering controls:

工程控制：

Use local exhaust ventilation, or other engineering controls to maintain airborne levels below exposure limit requirements or guidelines. If there are no applicable exposure limit requirements or guidelines, general ventilation should be sufficient for most operations. Local exhaust ventilation may be necessary for some operations.

使用局部排气通风，或其他工程控制，以保持空气中的水平低于暴露极限的要求或准则。如果没有适用的接触限值的要求。局部排气通风是必要的操作。

Personal protective equipment:

个人防护器材：

Eye and face protection:

眼睛和面部防护：

Use safety glasses. If there is a potential for exposure to particles which could cause eye discomfort, wear chemical goggles. If exposure causes eye discomfort, use a full-face respirator.

使用安全眼镜。如果有一个潜在的暴露，这可能会导致眼睛不适，戴化学眼镜。如果暴露引起眼睛不适，使用一个全面罩呼吸器。

Skin protection:

皮肤防护：

No precautions other than clean body-covering clothing should be needed.

除了清洁的衣服覆盖以外没有任何预防措施是可用的。

Hand protection:

手部防护：

Chemical protective gloves should not be needed when handling this material. Consistent with general hygienic practice for any material, skin contact should be minimized. Use gloves to protect from mechanical injury. Selection of gloves will depend on the task. Use gloves with insulation for thermal protection, when needed.

在处理时，不需要化学防护手套。可用一般卫生防护的材料，皮肤接触应尽量减少。用手套来保护防止机械损伤。手套的选择将视情况而定。在需要时使用隔热保温的手套。

Respiratory protection:

呼吸系统防护：

Chemical protective gloves should not be needed when handling this material. Consistent with general hygienic practice for any material, skin contact should be minimized. Use gloves to protect from mechanical injury. Selection of gloves will depend on the task. Use gloves with insulation for thermal protection, when needed.

在处理时，不需要化学防护手套。与一般卫生实践相一致的任何材料，皮肤接触应尽量减少。用手套来保护我机械损伤。手套的选择将取决于任务。在需要时使用隔热保温的手套。

Ingestion:

食入：

Use good personal hygiene. Do not consume or store food in the work area. Wash hands before smoking or eating.

养成良好的个人习惯。不要在工作区内食用或储存食物。吸烟或进食前洗手。

Environmental exposure controls:

环境防护：

Prevent from entering into soil, ditches, sewers, waterways and/or groundwater.

防止进入土壤、沟渠、下水道、水道和/或地下水。

Industrial hygiene:

工业卫生：

Handle in accordance with good industrial hygiene and safety practice.

Wash hands before breaks and at the end of workday.

No adverse effects are expected as a result of normal contact with them.

按照良好的工业卫生

休息前和工作结束时洗手。

没有不利影响，与他们的正常接触预计没有影响。

Section 9: Physical and chemical properties

9：理化性质：

9.1 Information on basic physical and chemical properties

9.1基本物理和化学性质的信息

| Appearance:外观 | Powder粉末 |
| --- | --- |
| Colour:颜色 | Off-white白色 |
| Odour:气味 | Odorless无味 |
| pH:PH值 | Not applicable.不可用 |
| Melting point:熔点 | No test data available.不可用 |
| Boiling point:沸点 | Not applicable不可用 |
| Specific Gravity:比重 | 1.2 - 1.8 Estimated (H2O = 1)1.2-1.8 |
| Density:密度 | No test data available.不可用 |
| Vapour pressure:蒸汽压力 | Not applicable.不可用 |
| Partition coefficient (n -octanol/water): 分配系数 | Not applicable. 不可用 |
| Solubility(ies):溶解度 | Insoluble in cold water.不溶于凉水 |
| Flash point:闪点 | No data available不可用 |
| Auto-ignition temperature:燃点 | No data available不可用 |
| Explosive properties:爆炸性 | No data available不可用 |
| Oxidising properties:氧化性 | No data available不可用 |

9.2 Other information

9.2其他信息

CPE is odourless nontoxic and white or pale yellow powder with an excellent resistance to weathering or age, ozone, chemicals and resistance to oil or flame. It is a colorable material with very good tenacity (flexible at -30 °C, a brittle temperature under -70 °C) and good miscible with various polymers due to its properties of both plastic and rubber. CPE 135 shows excellent resistance to very low temperature or high temperature.

CPE是无毒无味的白色或淡黄色粉末，具有优良的耐臭氧、化学药品和耐油阻燃，着色性，良好的材料韧性（在- 70°--C30°下的脆性温度）和与其他聚合物的相容性，而广泛应用于塑料和橡胶制品中。CPE 135具有优良的耐低温和耐高温性能。

Section 10: Stability and reactivity

10：稳定性和反应性

10.1 Reactivity

10.1反应性

It presents no significant reactivity hazards, by itself or in contact with water.

没有显著的反应危害

10.2 Chemical stability

10.2化学稳定性

Stable under recommended storage conditions.

在适当的存储条件下稳定。

10.3 Possibility of hazardous reactions

10.3危险反应的可能性

None hazardous reaction known. Hazardous polymerization Will not occur.

无危险反应。不会发生危险聚合。

10.4 Conditions to avoid

10.4禁止状况

Strong heating, open flames. Exposure to elevated temperatures can cause product to decompose

强加热，明火。暴露在高温下会导致产品分解

10.5 Incompatible materials

10.5不相容材料

No information available.

不可用

10.6 Hazardous decomposition products

10.6危险品的分解

Decomposition products depend upon temperature, air supply and the presence of other materials. Processing may release fumes and other decomposition products. At temperatures exceeding melt temperatures, polymer fragments can be released. Fumes can be irritating. Decomposition products can include and are not limited to: Aldehydes. Alcohols. Organic acids. Hydrogen chloride. Decomposition products can include trace amounts of: Hydrocarbons

分解产物取决于温度、空气供给和其他材料的存在。可能释放烟雾和其他分解产物。在熔融温度以上的温度可以释放聚合物碎片。烟雾会产生刺激性。分解产品可以包括但不限于：醇、有机酸、氯化氢。也可能包括痕量的碳氢化合物。

Section 11: Toxicological information

11：毒理学资料

11.1 Toxicokinetics, metabolism and distribution

11.1毒性动力学，代谢和分布

Based on best current information, there is no information known associated with this product.

没有与此相关的信息。

11.2 Information on toxicological effects

11.2毒理学效应

Acute toxicity:

急性毒性

The product is not expected to be acutely toxic.

Quantitative data on the acute oral/inhalation/dermal toxicity of this product are not available.

本产品的急性口服/吸入/皮肤毒性的定量数据是不可用的。

Skin corrosion/irritation:

皮肤腐蚀/刺激性：

No relevant information available.

不可用

Serious eye damage/irritation:

严重的眼损伤/刺激：

No relevant information available.

不可用

Respiratory or skin sensitization:

呼吸或皮肤过敏性：

No relevant information available.

不可用

Repeated dose toxicity:

重复毒性：

Repeated inhalation exposure may cause respiratory irritation and lung effects/injury. Impaired lung function and abnormal chest x-rays have been observed in humans repeatedly exposed to high levels of talc dust

反复吸入可能会导致呼吸道刺激和产生对肺的影响/损伤。在人类反复暴露于高浓度的灰尘下，已观察到肺功能的受损和胸部X射线异常

CMR effects (Carcinogenicity, Mutagenicity and Toxicity for Reproduction):

Cancer Information:

巨磁电阻效应（致癌、致突变性和生殖毒性）：

制癌信息：

Rats exposed for their lifetimes to very fine talc particles showed lung inflammation and fibrosis (both sexes) and lung tumors (females only). These effects are believed to be due primarily to overloading the normal respiratory clearance mechanism. Rats may be particularly susceptible to particle clearance overload, resulting in lung injury and tumors. An increase in spontaneously occurring adrenal tumors observed in male rats is of questionable relevance. No increases in tumors were observed in male or female mice.

暴露于非常细的滑石粉下表现出肺部炎症和纤维化（男女）和肺肿瘤（女性）。这些作用被认为主要是由于忽视了正常呼吸道清除机制。大鼠可能特别容易受到颗粒间隙过载，导致肺损伤和肿瘤。在雄性大鼠中观察到的铝肿瘤于此有相关性。在雄性或雌性小鼠中观察到的肿瘤没有增加。

Mutagenicity:No relevant information available.

诱变性：不可用

Toxicity for Reproduction:No relevant information available.

生殖毒性：不可用

STOT-single exposure and repeated exposure:

单次曝露和多次曝露：

Effects o, f Repeated , Exposure: Repeated inhalation exposure may cause respiratory irritation and lung effects/injury. Impaired lung function and abnormal chest x-rays have been observed in humans repeatedly exposed to high levels of talc dust.

暴露：反复吸入暴露可能会导致呼吸道刺激和肺的影响/损伤。多次接触高浓度滑石粉尘在人体中可观察到受损的肺功能和胸部X射线异常。

Section 12: Ecological information

12:生态信息

12.1 Toxicity

12.1毒性

The product is not expected to be acutely toxic to aquatic organisms.

Quantitative data on the acute fish/daphnia/bacteria toxicity of this product are not available.

定量数据对该产品的毒性是不可用的

12.2 Persistence and degradability

12.2持久性和降解性

This water-insoluble polymeric solid is expected to be inert in the environment.

Surface photo-degradation is expected with exposure to sunlight.

No appreciable biodegradation is expected.

在环境是惰性的

暴露于阳光表面降解

没有明显的生物降解。

12.3 Bioaccumulative potential

12.3生物积蓄潜力

No bio-concentration is expected because of the relatively high molecular weight (MW greater than 1000). In the terrestrial environment, material is expected to remain in the soil where it may be subject to wind dispersion. In the aquatic environment, material will sink and remain in the sediment.

因为相对较高的分子量（分子量大于1000），没有生物浓度预计。在陆地环境中，保持在土壤里可能会受风力分散。在水环境中，物质会下沉并停留在沉积物中。

Partition coefficient, n-octanol/water (log Pow): No test data available.

正辛醇/水分配系数，（辛醇）：没有可用的测试数据。

12.4 Mobility in soil

12.4迁移性

Based on best current information, there is no data known associated with this product

基于最佳的当前信息，没有已知与该产品相关的数据

12.5 Results of PBT and vPvB assessment

12.5评估结果

Based on best current information, there is no data known associated with this product.

基于最佳的当前信息，没有已知与该产品相关的数据

12.6 Other adverse effects

12.6其他影响

No other relevant information available.

没有相关材料

Section 13: Disposal considerations

13：处置注意事项

13.1 Waste treatment methods

13.1废物处理方法

Do not dump into any sewers, on the ground, or into any body of water. All disposal practices must be compliance with all federal, state and local laws and regulations. Regulations may vary in different locations. Waste characterizations and compliance with applicable laws are the responsibility solely of the waste generator. For unused & uncontaminated product, the preferred options include sending to a licensed, permitted recycler. Incinerator or other thermal destruction device. Landfill. If incineration is used, take precautions to guard against the formation of explosive dust air mixtures when handling combustible powders.

不要把废物，放在地上，或进入任何水体中。所有处理措施必须遵守所有的联邦、州和地方的法律和法规。位置不同规定可能会有所不同。废物的处理要遵守适用的法律。未使用的和未受污染的产品，优先选择包括森帝根据授权，允许回收。焚烧炉或其他热破坏装置。垃圾填埋.如果使用焚烧，当处理可燃粉末时采取预防措施防止爆炸性粉尘空气混合物的形成

Section 14: Transport information

14：运输信息

14.1 Land transport (ADR/RID/GGVSE)

14.1陆运

This product is not regulated as a hazardous material or dangerous goods for transportation.

本产品不受危险物质或危险货物运输的管制

14.2 Sea transport (IMDG-Code/GGVSee)

14.2海运

This product is not regulated as a hazardous material or dangerous goods for transportation.

本产品不受危险物质或危险货物运输的管制

14.3 Air transport (ICAO-TI/IATA-DGR)

14.3空运

This product is not regulated as a hazardous material or dangerous goods for transportation.

本产品不受危险物质或危险货物运输的管制

14.4 Additional information

14.4附加信息

This product is not hazardous for any transportation means.

Anyway, it should be stored in dry and ventilated room by its lot respectively.

Not allowed outdoor storage. Keep away from sunlight and humidity during its storage or transportation.

本产品不具有任何运输工具的危险性。

无论如何，它应存放在干燥和通风的地方。

不允许室外存储。在储存或运输过程中远离阳光和湿度。

Section 15: Regulatory information

15：监管信息

15.1 Safety, health and environmental regulations/legislation specific for the substance or mixture

15.1安全、健康和环境法规/特定的物质的法规

EU regulation:

欧盟法规：

Authorisations:No information available.

授权：不可用

Restrictions on use:No information available.

限制范围：不可用

EINECS:All the ingredients of the product are listed in the Inventory except of 64754-90-1

目录：参见64754-90-1

DSD (67/548/EEC):All the ingredients of the product are not listed in the Annex I.

Other chemical regulation:

USA - TSCA:All the ingredients of the product are listed in the Inventory.

Canada - DSL:All the ingredients of the product are listed in the Inventory.

Australia - AICS:All the ingredients of the product are listed in the Inventory.

Korea - ECL:All the ingredients of the product are listed in the Inventory.

Japan - ENCS:All the ingredients of the product are listed in the Inventory except of 14807-96-6.

China - IECSC:All the ingredients of the product are listed in the Inventory.

15.2 Chemical Safety Assessment

15.2安全评估

No Chemical Safety Assessment has been carried out for this product.

本产品已进行了化学安全性评价。

Section 16: Other information

16：其他信息

16.1 Revision Information:

16.1版本信息：

Date of the previous revision: Not applicable.

前修订日期：不可用。

Date of this revision: 01/12/2010       Revision summary: The first new SDS

本修订日期： 01/12/2010                  修订摘要：第一个

16.2 Abbreviations and acronyms

16.2缩写和首字母缩写词

CLP:EU regulation (EC) No 1272/2008 on classification, labelling and packaging of chemical substances and mixtures.

CAS:Chemical Abstracts Service (division of the American Chemical Society)

EINECS:European Inventory of Existing Commercial Chemical Substances.

RID:European Rail Transport.

IMDG:International Maritime Code for Dangerous Goods.

IATA:International Air Transport Association.

OSHA:The United States Occupational Safety and Health Administration.

TSCA:Toxic Substances Control Act, The American chemical inventory.

DSD:Dangerous Substance Directive (67/548/EEC).

IECSC:Inventory of existing chemical substances in China.

DSL:Domestic Substances List, The Canadian chemical inventory.

AICS:The Australian Inventory of Chemical Substances.

ECL:Existing Chemicals List, the Korean chemical inventory.

ENCS:Japanese Existing and New Chemical Substances.

16.3 Key literature references and sources for data

16.3主要文献参考文献和数据来源

ESIS IUCLID Dataset: European chemical Substances Information System.

HSDB: Hazardous Substances Data Bank.

ICSC: International Chemical Safety Cards.

GESTIS International Limit Values.

16.4 Relevant R-phrases and H-statements

16.4相关风险性短语

Not applicable.

This product does not meet the criteria for classification in any hazard class according to EU Regulation.

不可用

该产品不符合欧盟规定的任何危险类别的分类标准。

16.5 Training advice

16.5培训建议

No relevant information available.

不可用

16.6 Declare to reader

16.6向读者

The information in this Safety Data Sheet (SDS) was obtained from sources which we believe are reliable. However, the information is provided without any warranty, express or implied, regarding its correctness. The conditions or methods of handling, storage, use or disposal of the product are beyond our control and may be beyond our knowledge. For this and other reasons, we do not assume responsibility and expressly disclaim liability for loss, damage or expense arising out of or in any way connected with the handling, storage, use or disposal of the product. This SDS was prepared and is to be used only for this product. If the product is used as a component in another product, this SDS information may not be applicable. According to REACH Article 31(5), the SDS shall be supplied in an official language of the Member State(s) where the substance or mixture is placed on the market, unless the recipient Member State(s) concerned provide otherwise. It should also be noted that this SDS is applicable to the countries with English as an official language.

在这个安全数据表的信息是从可靠的来源得来。然而，提供的信息没有任何保证，明示或暗示。处理、储存、使用或出售该产品的条件或方法已超出我们的控制，并可能超出我们的能力。我们不承担相关责任并明确表示不承担损失责任，数据表的使用和处理存储所造成的损害或费用。十二烷基硫酸钠制备，并将仅用于本产品。如果该产品是在另一个产品中使用的一个组件，在这种情况下的数据可能不适用。自评量表应提供一个关闭的各成员国的通用语言。还应注意，这是适用于以英语作为官方语言的国家。

------------------------------End of the MSDS ----------------------------

结束